# COMPLEX COMPUTING ACTIVITY

COMPUTER VISION

NAME: HAFSA NAZ

ROLL No: 23-AI-56

BATCH 2023

DEPARTMENT: ARTIFICIAL INTELLIGENCE SECTION: A2

# Pose Estimation for Activity Detection (Standing vs Squatting)

# 1. Code Implementation Overview

# The project was implemented in Python using OpenCV and MediaPipe. It consists of three main tasks:

# Task 1: Real-time pose detection with skeleton overlay and angle extraction

# Task 2: Joint angle smoothing and visualization using plots

# Task 3: Accuracy evaluation using ground truth comparison

# The system processes a video frame-by-frame, detects human pose landmarks, computes joint angles, classifies activity, and displays results in real time.

# 2. Angle Computation Approach

# Pose landmarks were extracted using MediaPipe. Three joint angles were calculated:

# Knee (Hip–Knee–Ankle)

# Hip (Shoulder–Hip–Knee)

# Elbow (Shoulder–Elbow–Wrist)

# Key Feature Selection

# The knee angle was selected as the main feature because it shows clear variation between standing and squatting postures.

# 3. Rule-Based Classification Logic

# A simple threshold-based rule was designed:

# If knee angle > 140° → STANDING

# If knee angle ≤ 140° → SQUATTING

# This threshold was chosen based on observed joint behavior in the dataset:

# Standing → straight leg → high angle

# Squatting → bent knee → low angle

# 4. Signal Smoothing

# Joint angle data was noisy due to detection fluctuations. To stabilize results:

# Savitzky–Golay filter was applied

# Window size = 15, Polynomial order = 3

# This improved smoothness of angle curves and reduced false transitions.

# 5. Accuracy Results

# The predicted labels were compared with ground truth annotations.

# Total evaluated frames: 32

# Correct predictions: 29

# Incorrect predictions: 3

# Final Accuracy: 90.6%

# Error Analysis

# Misclassifications occurred mainly during transition frames where the subject was moving between standing and squatting.

# 6. Observations

# MediaPipe successfully detected pose in all frames

# Knee angle was the most reliable feature for classification

# Rule-based approach worked well for binary activity detection

# Smoothing significantly reduced prediction instability

# Errors mostly occurred during movement transitions

# 7. Demo Outputs

# 8. Conclusion

# This project demonstrates an efficient pose-based activity recognition system using simple geometric features. A rule-based classifier using knee angle achieved 90.6% accuracy, proving that lightweight approaches can effectively solve basic human activity recognition tasks.